Work report

From start to finish, the script is a preprocessing pipeline that takes (1) patient DICOM folders (to find the RTPLAN), (2) converted NIfTI data (CT + structure masks), and produces (3) a model-ready dataset per patient: sinogram artifacts, per-structure “Berlingo” montages, and a final stacked tensor X_montage.npy, plus a detailed JSON report (and quarantine handling when something is wrong).

At the top, we configure all paths and constants: where DICOMs live (INPUT_DIR), where the NIfTI conversion lives (NIFTI_ROOT), and where outputs must be written (OUTPUT_ROOT). Then you define global settings for saving (png/npy/both), proximity encoding parameters (how surface distance is mapped into [0..1]), PTV normalization bounds (Gy → 0..1), clustering tolerance for grouping PTV masks by dose, and the detector geometry (NX, NY, WIDTH_MM, HEIGHT_MM, SAD/SDD). This geometry defines the “camera” used later to project 3D masks into 2D detector stacks.

Next, the file sets up logging (file + console) and a toolbox of utility functions: time stamps, safe stats for arrays (_arr_stats), path/file metadata, JSON sanitization (convert numpy/path/NaN into JSON-safe values), and name normalization (normalize_name / _norm). There are also debug utilities to list all structure files for a patient, and verification helpers that check whether expected output files exist.

Then comes the PTV logic: it scans structure filenames, extracts a dose value from names like ptv_54_45 or ptv_69_96, rejects ambiguous cases (ranges, unions, “minus/opt/subtract”), and clusters candidates by dose with a tolerance. After clustering, the code can (depending on ptv_mode) assign masks into bins (ptv_br, ptv_ri, ptv_hr) and optionally “nest” them (so higher-dose tiers are included inside the lower tiers). It also writes metadata JSON (ptv_doses.json, ptv_channels.json) so you can audit what was detected.

The core imaging part is built around projection functions. We have two main projection modes:

- External entry/exit via GPUCastMaskEntryExit, producing two channels: where rays enter the body and where they exit (encoded using the same proximity mapping).

- All other masks via GPUCastMask with run_with_entry, producing either a binary fill or (in our current version) a proximity-encoded stack.
These project 3D CT-grid masks into stacks shaped [NY, NX, N_CP] (one detector image per control point), then reshape them into a single 2D “montage” [NY*N_CP, NX] for storage.

The BerlingoPipeline class ties everything together. In __init__, it stores paths and parameters, normalizes ptv_mode and save formats, and creates a quarantine folder inside the output root. It also provides helpers for RTPLAN parsing (extract angles, table positions, isocentre, CMW, meterset minutes, Tomo private tags), jaw/field-size estimation, and a cleanup method to optionally remove intermediate per-channel .npy after building X_montage.

The main workflow is process_patient(). It creates a patient-specific output directory, initializes a structured report dict, and then executes the pipeline in stages:

- Stage 0 (CT): Load CT from NIfTI (unless bundle_only) and record geometry + stats.

- Stage 1 (RTPLAN): Find and read RTPLAN, extract CPs, build sinogram, extract scalars (angles/tables/isocentre), estimate field size and jaws, save sinogram views, write a compact plan summary, and save an “absolute training bundle” (angles, cp durations derived from BeamMeterset+ΔCMW, etc.). If mode is bundle_only, it stops here safely.

- Stage 2 (Structures): Locate structures folder, optionally dump debug list of all structure files, then load masks for expected organs and grouped organs. Loading is done through choose_or_quarantine_roi() which detects multi-structure-set conflicts and can quarantine if strict mode finds mismatched content.

- Stage 3 (PTV): If enabled, detect and cluster PTV masks, bin them (br/ri/hr), union them on the CT grid, apply nesting rules, and write PTV metadata.

- Stage 4 (Projections + Save): Compute external_entry and external_exit, then project every other expected structure using proximity encoding. Each channel gets saved as berlingo_<name>.npy (and/or png) as a montage, with per-channel stats in the report.

- Stage 5 (Build X): Build X_montage.npy by loading the saved montages from disk in the exact order OAR_AND_GROUPS_ORDER, producing an array shaped [C, NY*N_CP, NX].

- Stage 6 (Optional cleanup): Delete intermediate berlingo_<key>.npy files if configured.

- Stage 7 (Final): Write patient_report.json and finish with status ok. Any exception triggers the fatal handler: it writes a report if possible and moves the patient output to quarantine.

Finally, at the bottom, the __main__ section acts as the entry point: it instantiates the pipeline with the configured paths and modes, enumerates patient folders, and runs process_selected(..., n_jobs=3) (or process_all). That’s the “batch driver” that lets you run a subset or the full dataset, optionally in parallel.

If you want, I can also summarize it as a simple “dataflow diagram” (inputs → intermediate artifacts → outputs) so it’s easier to reason about what files get created and where.